# Designed a battery

- TP14 (for charging )

- 3v to 7v converter

- Li-ion Cell

- Cell case

Circuit schematics

Features

- Change cell

- Charge

- Solar (A/N)

- System to identify dead cells (N/A)

Why we made Our Own

- Cost

- Size

- Cell change functionality

- Solar (N/A)

# Lora module range test

- 2x Esp 32

- 2x Lora sx1278 module

- 1x Usb cable for programming

- 1x Led

- Breadboard

- Jumper wires

Sender Wire Connection’s

| Lora | Esp32 |
| --- | --- |
| GND | GND |
| 3.3V | 3.3V |
| RST | PIN 4 |
| D1O0 | PIN 13 |
| NSS/CS/SS | PIN 5 |
| MOSI | PIN 23 |
| MISO | PIN 19 |
| SCK | PIN 18 |

Receiver wire connection’s

| Lora | Esp32 |
| --- | --- |
| BOTH SIDE GND | GND |
| 3.3V | 3.3V |
| RST | PIN 4 |
| D1O0 | PIN 13 |
| NSS/CS/SS | PIN 5 |
| MOSI | PIN 23 |
| MISO | PIN 19 |
| SCK | PIN 18 |
| LED | PIN 2 |

Sender side code

#include <SPI.h>

#include <LoRa.h>

#define lora_ss 5

#define lora_rst 4

#define lora_dio0 13

void setup() {

serial.begin(115200);

while (!serial);

// initialize lora module

lora.setpins(lora_ss, lora_rst, lora_dio0);

if (!lora.begin(433e6)) {  // change to your lora module's frequency

serial.println("lora init failed. check your connections.");

while (true);

}

serial.println("lora sender initialized");

}

void loop() {

serial.println("sending packet...");

lora.beginpacket();

lora.print("hello");

lora.endpacket();

delay(1000);  // wait 1 second before sending the next packet

}

Receiver side code

#include <SPI.h>

#include <LoRa.h>

#define LORA_SS 5

#define LORA_RST 4

#define LORA_DIO0 13

#define LED_PIN 2

void setup() {

Serial.begin(115200);

while (!Serial);

pinMode(LED_PIN, OUTPUT);

digitalWrite(LED_PIN, LOW);

// Initialize LoRa module

LoRa.setPins(LORA_SS, LORA_RST, LORA_DIO0);

if (!LoRa.begin(433E6)) {  // Change to your LoRa module's frequency

Serial.println("LoRa init failed. Check your connections.");

while (true);

}

Serial.println("LoRa Receiver Initialized");

}

void loop() {

int packetSize = LoRa.parsePacket();

if (packetSize) {

String received = "";

while (LoRa.available()) {

received += (char)LoRa.read();

}

Serial.print("Received: ");

Serial.println(received);

// Blink LED when data is received

digitalWrite(LED_PIN, HIGH);

delay(100);  // LED on for 100 ms

digitalWrite(LED_PIN, LOW);

} else {

// No data received, keep LED off

digitalWrite(LED_PIN, LOW);

}

}

Result of test : - 315m in open area

# Lora module Temperature sensor

Sender Wire Connection’s

| Lora | Esp32 |
| --- | --- |
| GND | GND |
| 3.3V | 3.3V |
| RST | PIN 4 |
| D1O0 | PIN 13 |
| NSS/CS/SS | PIN 5 |
| MOSI | PIN 23 |
| MISO | PIN 19 |
| SCK | PIN 18 |
| DHT22 | PIN 27 |

Receiver wire connection’s

| Lora | Esp32 |
| --- | --- |
| BOTH SIDE GND | GND |
| 3.3V | 3.3V |
| RST | PIN 4 |
| D1O0 | PIN 13 |
| NSS/CS/SS | PIN 5 |
| MOSI | PIN 23 |
| MISO | PIN 19 |
| SCK | PIN 18 |
| LED | PIN 2 |

Sender side code

#include <SPI.h>

#include <LoRa.h>

#include <DHT.h>

#include <BluetoothSerial.h>

// LoRa pins

#define LORA_SS 5

#define LORA_RST 4

#define LORA_DIO0 13

// DHT22 pin and type

#define DHTPIN 27

#define DHTTYPE DHT22

DHT dht(DHTPIN, DHTTYPE);

// Bluetooth object

BluetoothSerial SerialBT;

void setup() {

Serial.begin(115200);

while (!Serial);

// Initialize DHT22

dht.begin();

// Initialize LoRa

LoRa.setPins(LORA_SS, LORA_RST, LORA_DIO0);

if (!LoRa.begin(433E6)) {  // Change frequency based on your module

Serial.println("LoRa init failed. Check connections.");

while (true);

}

// Set LoRa parameters for maximum range

LoRa.setSpreadingFactor(12);  // Maximum spreading factor

LoRa.setSignalBandwidth(125E3);  // Narrow bandwidth

LoRa.setCodingRate4(8);  // Maximum coding rate

Serial.println("LoRa Sender Initialized");

// Initialize Bluetooth

if (!SerialBT.begin("ESP32_Sender")) {

Serial.println("Bluetooth init failed.");

while (true);

}

Serial.println("Bluetooth Initialized as 'ESP32_Sender'");

}

void loop() {

// Read temperature and humidity from DHT22

float temperature = dht.readTemperature();

float humidity = dht.readHumidity();

// Handle DHT22 errors

if (isnan(temperature) || isnan(humidity)) {

Serial.println("Failed to read from DHT sensor!");

return;

}

// Prepare sensor data

String data = "Temp: " + String(temperature) + "°C, Hum: " + String(humidity) + "%";

// Send data over LoRa

Serial.println("Sending packet: " + data);

LoRa.beginPacket();

LoRa.print(data);

LoRa.endPacket();

// Send data over Bluetooth

SerialBT.println(data);

delay(2000);  // Wait 2 seconds before next iteration

}

Receiver side code

#include <SPI.h>

#include <LoRa.h>

#include <BluetoothSerial.h>

// LoRa pins

#define LORA_SS 5

#define LORA_RST 4

#define LORA_DIO0 13

// LED pin

#define LED_PIN 2

// Bluetooth object

BluetoothSerial SerialBT;

void setup() {

Serial.begin(115200);

while (!Serial);

// Initialize LED

pinMode(LED_PIN, OUTPUT);

digitalWrite(LED_PIN, LOW);

// Initialize LoRa

LoRa.setPins(LORA_SS, LORA_RST, LORA_DIO0);

if (!LoRa.begin(433E6)) {  // Change frequency based on your module

Serial.println("LoRa init failed. Check connections.");

while (true);

}

// Set LoRa parameters for maximum range

LoRa.setSpreadingFactor(12);  // Maximum spreading factor

LoRa.setSignalBandwidth(125E3);  // Narrow bandwidth

LoRa.setCodingRate4(8);  // Maximum coding rate

Serial.println("LoRa Receiver Initialized");

// Initialize Bluetooth

if (!SerialBT.begin("ESP32_Receiver")) {

Serial.println("Bluetooth init failed.");

while (true);

}

Serial.println("Bluetooth Initialized as 'ESP32_Receiver'");

}

void loop() {

// Check for LoRa packet

int packetSize = LoRa.parsePacket();

if (packetSize) {

String received = "";

while (LoRa.available()) {

received += (char)LoRa.read();

}

Serial.print("Received via LoRa: ");

Serial.println(received);

// Blink LED to indicate data reception

digitalWrite(LED_PIN, HIGH);

delay(100);

digitalWrite(LED_PIN, LOW);

// Send data to Bluetooth

SerialBT.println(received);

}

// Check for Bluetooth input

if (SerialBT.available()) {

String bluetoothMessage = "";

while (SerialBT.available()) {

bluetoothMessage += (char)SerialBT.read();

}

Serial.print("Received via Bluetooth: ");

Serial.println(bluetoothMessage);

// Echo the Bluetooth message back

SerialBT.println("Echo: " + bluetoothMessage);

}

delay(10);

}

Result of test : - Successful And data can be extracted via Bluetooth serial terminal on android.